Adaptive Robust CBF for Quadrupedal Safety —
Project Configuration Snapshot

Update for Prof. Cosner — 2026-05-07 / Target venue: CoRL 2026 (abstract May 25, paper May 28)

# 1. Project framing

Working title: Adaptive Robustness Margins for Control Barrier Functions via Privileged-Observation Reinforcement Learning — A Quadrupedal Safety-Filter Case Study.

Core claim: a privileged-observation RL teacher can adapt the four robust-CBF parameters (α, φ, a, c) online and Pareto-dominate hand-tuned plain / ISSf / TISSf baselines on a combined fall + stuck failure metric, without sacrificing the safe-set forward-invariance structure. Inspired by Rapid Motor Adaptation (RMA): teacher conditions on privileged observations during training; a student replays it from LiDAR + base velocity + history for deploy.

Closest published baseline: SHIELD (Yang et al. 2025, arXiv:2505.11494) — same exponential-smoothed SDF form, real LiDAR + cluster-fit-cylinder, Unitree G1 humanoid. Differentiation: (1) per-step multi-param adaptation vs their per-episode single-α calibrated by Freedman's inequality; (2) four robust slacks tied to specific uncertainty classes (Molnar / Kolathaya / Dean / boundary) plus B-fixed-X ablations as paper Table 2; (3) arbitrary obstacle shapes once perception pipeline (v2.12) lands. Cosner is a co-author on SHIELD, so this is positioned as related work, not a conflict.

# 2. Pipeline at a glance

| Hardware | Unitree Go2 quadruped + Livox Mid-360 LiDAR. Go2 runs ROS 2 Humble on Jetson Orin. |
| --- | --- |
| Sim | Isaac Lab on RTX 5090 (lab box, SSH chrisliang@130.64.84.163). Built on Isaac-Velocity-Flat-Unitree-Go2-v0 as locomotion base. |
| Locomotion checkpoint | Off-the-shelf flat-trained policy (unitree_go2_flat/2026-04-15_19-24-45/exported/policy.pt). Native fall ~0.5%; in our CBF env at B0 ~20% due to 50 Hz command stream vs 10 s held training commands. Treated as a frozen, drop-in component (works with any user's locomotion stack). |
| Robot model | Single-integrator (ẋ_robot = u). L_g h ≠ 0 → standard CBF works. HOCBF only needed if we move to torque-level control (not on roadmap). |
| Outer loop | RL teacher emits robust-CBF params (α, φ, a, c) at 50 Hz; QP wraps the off-the-shelf locomotion controller; output goes to walking_bridge → SportClient.Move. |
| ROS 2 package | Minimal extraction from semantic-safety: walking-bridge + LiDAR occupancy grid only. YOLO / cameras / safety filter from upstream all stripped. Source files in place; not yet built or tested on hardware. |

# 3. CBF formulation and four-parameter mapping

Safe-set h(x) is built from per-shape analytical SDFs (boxes, cylinders, walls) combined via Eq. 19 multi-obstacle SDF + Eq. 20 exponential smoothing. Robot footprint enters as Minkowski expansion by 0.15 m. The safety QP is solved closed-form as a half-space projection on GPU — no external solver in the hot loop.

Constraint enforced (audited 2026-05-07 against cbf_go2_env.py:168-230):

L_g h · u_safe  ≥  −α(h − c)  +  φ · ‖L_g h‖²  +  a

Per-parameter mapping to the literature:

| Param | Range (v2.6/v2.10) | Wide (v2.11) | Role | Reference |
| --- | --- | --- | --- | --- |
| α | [0.1, 5.0] | unchanged | Class-K slope on shifted h(x)−c; absorbs model / tracking error. | Molnar 2021 |
| φ | [0.0, 5.0] | unchanged | Coefficient on ‖L_g h‖² (RHS); absorbs actuation uncertainty. | Kolathaya 2018 (ISSf) |
| a | [0.0, 1.0] | [0.0, 3.0] | Additive RHS slack; absorbs state-independent measurement uncertainty. | Dean 2019 |
| c | [0.0, 0.5] | [0.0, 1.0] | Inward shift of safe-set boundary; boundary correction when h_lidar underestimates. | Boundary-correction |
| b | unused | unused | 5th action slot reserved for input-dep slack b·‖u‖; needs SOCP solver — not active. | Dean 2019 |

Action space is 5-D (α, φ, a, b, c) with only 4 active. Currently a known dead-weight gap: training has zero measurement noise, so a has no gradient signal and likely settles to a constant. v2.12 NoisyPerception OOD env will exercise it.

Two L_f h items distinguished — (1) obstacle-drift term added in v2.11 behind USE_LFH_OBSTACLE_DRIFT flag (∂h/∂p_obs · v_obs added to RHS, ~15 lines, no solver change; matters most for FastObstacles); (2) HOCBF for robot model, only relevant if we switch from single- to double-integrator dynamics (kept as honest paper limitation).

# 4. Privileged observation and teacher network

| Priv obs (PRIV-2) | 8207-D = 15 dynamics + 8192 occupancy grid (2 frames × 64 × 64 × 0.1 m, ego-centric, 6.4 m FOV). |
| --- | --- |
| Architecture | priv → CNN [Conv 2→16 s=2, 16→32 s=2, Linear → 64], dyn MLP [15 → 64] → concat → Linear 128 → 12 → Z(12) → π_teacher (128, 5). |
| Bottleneck | Z is 12-D — the RMA latent. get_z(obs) is exposed; student in Wk3 will reproduce Ẑ from (LiDAR, base_vel, history) via LSTM or 1-D conv, then feed frozen π_teacher. |
| Output | 5-D action (α, φ, a, b, c); b unused at QP. |
| u_des | Never enters the network. Sidecar to the CBF; only used in reward via ‖u_safe − u_des‖² penalty. |

# 5. PPO regularization (v2.6 working recipe — frozen)

| Optimizer | AdamW, weight_decay = 1e-5 (monkey-patched on OnPolicyRunner.__init__). |
| --- | --- |
| Entropy coef | 0.005 — load-bearing fix (5× v2.5's 0.001 prevented action-std collapse). |
| Action-rate penalty | −0.005 · ‖Δa‖² (gentler than v2.5's −0.01). |
| Smoothness mechanisms | Orthogonal: weight_decay smooths input → output map; action-rate smooths in time. |
| Episode length | 20 s; resampling controls planner switching cadence. |

# 6. Reward stack (v2.10 on disk = v2.9b reward; v2.6 ckpt was trained on v2.6 stack)

| Term | Weight | Type | Notes |
| --- | --- | --- | --- |
| collision | −100 | terminal | Obstacle contact (per-shape SDF < 0). |
| base_contact_penalty | −100 | terminal | Fall — REWARD-2 NEW; was missing before v2.9b. |
| stuck | −2.0 / step | shaping | Triggers when ‖v_xy‖ < 0.15 m/s — REWARD-2 NEW. |
| infeasibility | −10 / step | shaping | QP infeasibility marker. |
| u_safe_deviation | −0.1 · ‖u_safe − u_des‖² | shaping | Per-step. |
| proximity | −0.5 · exp(−min_sdf/0.5) | shaping | REWARD-2 halved from −1.0. |
| action_rate | −0.005 · ‖Δa‖² | shaping | Smooths CBF params over time. |
| u_safe_rate | (unregistered) | — | Function lives in code but unwired. Reserved for REWARD-3 if needed. |

v2.6 stack (paper baseline ckpt) had proximity at −1.0, no base_contact_penalty, no stuck term. REWARD-2 (v2.9 → v2.9b → v2.10) is the retune that flipped compound to a WIN but trades off DR width — see §10.

# 7. Domain randomization (v2.10 on disk = v2.6 narrow DR; reverted 2026-05-07)

| Axis | Knob | Train range | OOD eval range |
| --- | --- | --- | --- |
| Friction | static / dynamic | (0.30, 1.20) / (0.20, 1.00) | (0.15, 1.50) / (0.10, 1.30) |
| Disturbance | force / torque | ±10 N / ±2 Nm | ±18 N / ±3.5 Nm |
| COM offset | xy / z | ±5 cm / ±3 cm | ±8 cm / ±5 cm |
| Obstacle motion | max_speed | 0.2 m/s | 0.4 m/s |
| Obstacle density | separation_buffer | 0.4 m | 0.2 m |
| Obstacle count | K_actual ∈ [0, 20] uniform per-reset | — | — |
| Obstacle pool | K_MAX = 20 (8 cubes, 6 cylinders, 4 walls, 2 rect boxes, 0.20–2.0 m) | — | — |
| Moving obs | ~50 % drift constant velocity per episode (±0.5 m/s per axis) | — | — |

v2.7 widened DR aggressively (friction +50%, force +80%, motion 5×) at 3000 iters → under-converged. v2.8 / v2.9 / v2.9b widened mildly at 5000 iters → uniformly raised absolute combined eval values. v2.10 reverted to v2.6 narrow DR; v2.11 layers variable obstacle-motion DR (per-episode v_obs ∈ [0, 0.4] m/s) plus bimodal resample DR on top.

# 8. Multi-planner training mix

Working baseline (v2.6 ckpt — paper baseline):

| Mix | smooth_goal 0.40 / waypoint 0.25 / mpc 0.20 / legacy_goal 0.05 / walk 0.05 / adversarial 0.05 |
| --- | --- |
| Resample | resampling_time_range = (10, 10) → 1 mid-episode switch per 20 s episode. |

On-disk env_cfg (v2.10):

| Mix | smooth_goal 0.45 / waypoint 0.30 / mpc 0.20 / legacy_goal 0.05 — PLANNER-2b dropped walk + adversarial. |
| --- | --- |
| Resample | (100, 100) → locked per episode (PLANNER-2a). Deployment-realistic. |

v2.11 (live):

| Resample DR | Bimodal: P=0.5 → uniform [5, 15] s (mid-switch episode); P=0.5 → 100 s (locked episode). Restores v2.6's stuck-recovery regularizer that PLANNER-2a stripped in v2.8 / v2.10. |
| --- | --- |
| Eval | Always locked (100 s). The training/eval mismatch is the point: training mid-switch teaches intrinsic recovery; locked-planner eval verifies it transfers to deploy. |

# 9. Headline eval matrix — v2.6 (paper baseline)

| Eval | Type | Margin (BR vs best baseline, combined fall+stuck) |
| --- | --- | --- |
| In-distribution | mixed | +6.9 pp WIN |
| In-dist + locked-planner eval (PLANNER-2a only) | mixed | +10.5 pp WIN (free win on v2.6 ckpt) |
| Slippery | priv-obs (continuous friction shift) | +5.6 pp WIN |
| DensePack | scene-only (h(x) symmetric) | +0.6 pp tie |
| HighDisturbance | priv-obs (episodic force/torque) | +9.0 pp WIN |
| FastObstacles | priv-obs (grid history) | +10.6 pp WIN |
| HeavyCOM | priv-obs (startup COM bias) | +5.9 pp WIN |
| RealisticCompound | all 5 modest pushes simultaneously | −0.3 pp tie |

Pattern: WINs where the teacher has asymmetric privileged information (friction, force, COM, velocity history); ties where the playing field is symmetric (DensePack — same h(x) for both methods) or compositional (RealisticCompound — joint high-tail untrained).

# 10. Version progression (Wk2: May 4 – 10)

| Version | Status | Headline result | Take-away |
| --- | --- | --- | --- |
| v2.6 | ✓ paper baseline holds | In-dist +6.9 pp; 5 OOD WINS / 1 compound TIE. | Narrow DR + entropy 0.005 + mid-switch planner. Action-std 0.42, base_contact 5.8 % (lowest). |
| v2.7 | abandoned (05-06) | In-dist −0.5 pp; Slippery −11.4 pp. | Wider DR + 3000 iters → under-converged checkpoint. |
| v2.8 | abandoned (05-06 eve) | In-dist 49.3 % vs v2.6's 30.6 % (+18.7 pp regression). Stuck 7.5 % → 22.8 %. | PLANNER-2a (locked train) + 2b (drop walk/adv) + mild DR widen → removed two regularizers; policy never learned to recover from CBF deflection. |
| v2.9 | abandoned (05-07) | Stuck FIXED 22.8 → 7.7 %; fall WORSENED 26.5 → 40.8 %. −1.2 pp LOSS. | REWARD-2 (base_contact_penalty −50, stuck −2.0, proximity −1.0 → −0.5). Reward-shaping direction was right; magnitude weak. |
| v2.9b | abandoned (05-07 late) | In-dist −0.5 pp tie; 5 OOD WINS; Compound flipped to WIN (+1.7 pp); DensePack −1.7 pp. | Single-knob retune base_contact_penalty −50 → −100. Pushed fall down (40.8 → 35.0 %) but stuck rose (7.7 → 12.9 %); compound win is real. |
| v2.10 | ✓ done (05-07 very late); fallback ckpt archived | In-dist +9.9 pp WIN (combined 0.343); 4W / 2T / 1L. HeavyCOM −7.8 pp LOSS. | Reverted DR + OOD ranges to v2.6; kept PLANNER-2a/2b + REWARD-2 retune. HeavyCOM mid-switch diagnostic: combined dropped 0.485 → 0.397 from eval-time planner regime alone → smoking gun: PLANNER-2a (locked training) was the dominant HeavyCOM regression cause. |
| v2.11 | RUNNING (launched 05-07 overnight, ETA ~9 h) | Pending. | Bimodal resample DR + variable obstacle-motion DR + L_f h obstacle-drift term + WIDE_PARAM_RANGES (a [0,3], c [0,1]) + B-fixed-{α,φ,a,c} eval modes + 12 CBF training health stats + parallel 2-up eval. No new reward terms (DR-implicit shaping). |

Decision criteria for v2.11 ship: in-dist combined ≤ 0.31 AND HeavyCOM margin ≥ +3 pp AND compound holds → v2.11 ships as paper baseline; tag + archive ckpt; proceed to Wk3 student distillation OR v2.12 perception rebuild. Fallback: v2.6 + locked-planner-eval (already +10.5 pp in-dist) is a defensible paper.

# 11. Remaining roadmap (CoRL 2026 — abstract May 25, paper May 28)

| Week | Focus | Status |
| --- | --- | --- |
| Wk1 (Apr 28 – May 3) | Multi-obstacle + varying-radius retrain + OOD eval | ✓ done; v1 mean rank 1.5 |
| Wk2 (May 4 – 10) | v2 architecture + headline win | v2.6 closed (5 wins / 1 tie); v2.7-v2.9b abandoned; v2.10 done, fallback. v2.11 running. |
| Wk3 (May 11 – 17) | Student distillation + paper draft | Not started; gated on ≥ v2.6 baseline. |
| Wk4 (May 18 – 25) | Sim-to-real prep, polish, submit | Not started. |

Key items remaining:

v2.11 — bimodal resample DR + L_f h drift + wide param ranges + B-fixed-X ablations. Tests bimodal as single attribution against v2.10's HeavyCOM regression.

v2.12 — Realistic perception in 4 layers: L1 grid-based h via distance transform (train/deploy parity); L2 censored grid (raycast occlusion + range falloff); L3 Gaussian noise on grid + Isaac-CBF-Go2-NoisyPerception-v0 OOD env; L4 arbitrary obstacle shapes (irregular meshes, free once L1 lands). ~4-5 days.

Paper Table 2 — 4-axis fixed-param ablation: BR vs B-fixed-{α,φ,a,c}, each param matched to its OOD axis (DensePack, HighDisturbance, NoisyPerception, HeavyCOM/FastObs).

Stress curve (paper figure): sweep one priv-obs axis from training-edge to ~2× past in 4–5 steps. Shows whether BR's edge grows with difficulty.

Wk3 student distillation: re-enable LiDAR (multi-mesh K>1 obstacles), adapter (LSTM or 1-D conv over history) → Ẑ ∈ R¹², stage-2 supervised on teacher rollouts.

Wk4 sim-to-real: TorchScript export of student adapter + π_teacher; walking_bridge Sport-Mode → low-level Unitree motor API; 1-2 hardware demos (stretch).

# 12. Active risks (likelihood × impact)

| Risk | L | I | Mitigation / current state |
| --- | --- | --- | --- |
| Adaptation claim doesn't land | L | H | v2.6 holds (5 wins / 1 compound tie / +6.9 pp; +10.5 pp with locked-eval). v2.9b actually won compound (+1.7 pp) where v2.6 tied — REWARD-2 fixes something on compositional stress. v2.11 testing DR-implicit retune. |
| Sim-to-real gap on LiDAR | H | M | Inject matching noise during student training (v2.12 L3); v2.12 L1 closes train-deploy h-pipeline parity gap. |
| Reward tuning hell | M | M | Bounded ablation budget; user preference for DR-implicit shaping over hand-crafted reward terms (cleaner paper claim). |
| Teacher–student unstable | M | M | Fall back to end-to-end PPO. Z bottleneck (12-D) keeps the surface small. |
| Sim-to-real gap on dynamics | M | M | Robust CBF absorbs via φ; sim-only paper still defensible. |
| Locomotion fails on real floors | M | M | Off-the-shelf flat-trained loco; gap absorbed by teacher's robust CBF. |
| CBF-QP infeasible at deploy | L | L | Soft constraints; damping fallback. |
| Hardware breaks | L | M | Sim-only paper still submittable. |

Drop list (cut in this order if timeline tightens):

Hardware demo (Wk4 stretch) → sim-only paper.

L_f h obstacle-drift term + HOCBF → keep as honest future-work limitations.

v2.12 NoisyPerception env → ship without it; flag a as future-work in limitations (loses 1 of 4 ablation rows).

v2.11 (DR-implicit shaping + B-fixed-X) → ship v2.10 + 5-axis OOD as headline.

REWARD-3 (u_safe_rate registration) → ship v2.10 with whatever fall rate it lands at.

Further reward tuning after v2.10 → ship v2.10 result.

v2.10 / v2.11 retrain → ship v2.6 + locked-planner-eval as the headline.

Per-planner breakdown eval → headline + near-OOD covers the claim.

# 13. Open questions for discussion

Is bimodal resample DR (50/50 mid-switch / locked) defensible as a paper claim, or does the training/eval mismatch read as an artifact?

Paper Table 2 — is per-axis fixed-param ablation (BR vs B-fixed-α/φ/a/c) the right ablation, or should we instead show param-trajectory plots from rollouts (showing the policy actually varies the params with the obs)?

If v2.11 ships, do we go straight to v2.12 perception rebuild (4-5 days, hits the a-slot dead-weight gap and the deploy-realism gap simultaneously), or pivot to Wk3 student distillation?

For the SHIELD comparison, do we run it as an actual baseline in our eval matrix or just position as related work?

How aggressive on hardware demo for Wk4 — chair + doorway, or sim-only and full polish?

Generated 2026-05-07. Source: PROGRESS.md (1191 lines), CLAUDE.md, docs/class_paper/main.tex. v2.11 results pending (~9 h ETA).